Life Cycle Assessment of Small-Scale Tofu Production in Semarang City: Hotspot Analysis Using CML-IA and AWARE Methods

Abdul Jabbar1, Putri Alifa Kholil1* and Desiana Fitri Awati1

1 Study Program of Environmental Science, Faculty of Mathematics and Natural Sciences, Universitas Negeri Semarang, Semarang, Indonesia

*E-mail: putrialifa@mail.unnes.ac.id

Abstract. The tofu industry is a vital food-processing sector in Indonesia, but its production requires high water and energy inputs and generates organic wastewater. This study evaluated the potential environmental impacts of small-scale tofu production in Semarang City using a gate-to-gate Life Cycle Assessment with a functional unit of 1 kg of tofu. The system boundary covered washing, soaking, grinding, boiling, filtration, coagulation, and molding and packing. Impact assessment was performed in SimaPro 10.3 using the CML-IA Baseline method for Global Warming Potential (GWP), Ozone Layer Depletion (ODP), Freshwater Aquatic Ecotoxicity (FAET), Acidification (AP), and Eutrophication (EP), and water scarcity was additionally assessed with the AWARE method. The boiling and coagulation stages were the dominant environmental hotspots. Per kilogram of tofu, total GWP was 1.84 × 10⁻¹ kg CO₂ eq and was governed by firewood use at the boiling stage (88.0%), while FAET (1.30 × 10⁻² kg 1,4-DB eq) was distributed across boiling (34.7%), coagulation (29.5%), and grinding (24.2%). ODP (2.53 × 10⁻¹⁰ kg CFC-11 eq) and AP (1.27 × 10⁻⁴ kg SO₂ eq) were led by the coagulation stage, whereas EP (1.75 × 10⁻⁴ kg PO₄³⁻ eq) was driven by boiling and by the whey discharged as emission to water at coagulation. After normalization, GWP showed the highest value (3.66 × 10⁻¹⁴), followed by FAET (2.50 × 10⁻¹⁴) and EP (1.32 × 10⁻¹⁴), indicating that firewood-related climate impact and freshwater ecotoxicity are the foremost environmental priorities. The AWARE assessment yielded a total water-scarcity impact of 569 m³ world eq, dominated by the boiling stage (51.7%) through the embedded water footprint of firewood. Practical improvement strategies include organic wastewater treatment with biogas recovery, water reuse, fuel substitution, and improved thermal efficiency. This study provides process-level evidence to support cleaner production in small-scale urban tofu industries in Semarang City.

Keywords: Life Cycle Assessment; Tofu Production; CML-IA Baseline; AWARE; Environmental Hotspot.

1. Introduction

Tofu (tahu) is a staple plant-based protein in Indonesia, consumed at roughly 0.16 kg per capita per week and produced largely by household-scale and small and medium-sized enterprises (SMEs) [1]. This sustained demand generates large volumes of high-strength organic wastewater rich in dissolved proteins, suspended solids, and soybean residues, which are frequently discharged with little or no treatment [2, 3]. Such effluents are a recognized driver of freshwater degradation: the food-processing sector is among the leading contributors to surface-water pollution in Indonesia, with untreated organic discharge elevating biochemical oxygen demand (BOD) and chemical oxygen demand (COD) in receiving rivers well beyond regulatory limits [4]. In dense urban settings such as Semarang City, where tofu SMEs typically operate close to riverine systems with limited treatment infrastructure, the ecological significance of process-level liquid waste is amplified. Beyond liquid effluents, however, tofu processing is also energy-intensive, depending heavily on firewood combustion at the boiling stage; a balanced environmental assessment must therefore weigh both water-related and energy-related burdens rather than either alone.

Life Cycle Assessment (LCA) standardized under the ISO 14040 and ISO 14044 framework, is a rigorous approach for quantifying environmental burdens across a defined production system and for identifying the stages that disproportionately drive impacts [5]. LCA has been applied to Indonesian tofu production, but with recurring methodological gaps. Rosyidah et al. [6] and Nugroho et al. [7] focused mainly on greenhouse-gas emissions and identified firewood-fired boiling as the dominant emission source; Sari et al. [8] applied CML-IA in a cradle-to-gate study but did not resolve freshwater ecotoxicity at the stage level; and Hartini et al. [9] coupled LCA with life cycle costing and confirmed freshwater ecotoxicity and eutrophication as leading categories. However, none of these gate-to-gate Indonesian studies combined the CML-IA Baseline method with a dedicated, stage-resolved water-scarcity assessment [10]. Critically, the AWARE water-scarcity method has not been applied to Indonesian tofu production, and no LCA has been reported for Semarang City, where urban water dynamics may yield burden profiles distinct from rural production sites [11].

This study addresses these gaps by conducting a gate-to-gate LCA of SME-scale tofu production in Semarang City with four objectives: (1) to quantify impacts across five CML-IA Baseline midpoint categories using SimaPro 10.3; (2) to assess water scarcity with the AWARE method; (3) to characterize stage-level material and energy flows following standardized life cycle inventory guidance; and (4) to identify environmental hotspots and propose targeted, source-oriented improvement strategies for urban tofu SMEs, consistent with cleaner-production and circular-economy principles.

2. Methods

This research used a quantitative Life Cycle Assessment (LCA) approach to evaluate the environmental impacts of tofu production at an SME in Semarang City. The assessment followed SNI ISO 14040:2016 and SNI ISO 14044:2016 and comprised four sequential phases: (1) goal and scope definition, (2) life cycle inventory analysis, (3) life cycle impact assessment, and (4) interpretation. Primary data were obtained through direct field measurements, structured interviews with the facility operator, and on-site documentation during a complete production batch. Secondary data were sourced from peer-reviewed literature and the Ecoinvent v3 database within SimaPro 10.3.

2.1 Goal and scope definition

This study aims to assess the environmental impacts of tofu production at the facility level, focusing on water use, energy consumption, and waste generation at each production stage. The functional unit was established as 1 kg of tofu produced, consistent with functional units used in comparable LCA studies of tofu and soy-based food processing [8]. All inventory data were initially collected on a per-batch basis representing one complete production day, with 1,400 kg of soybeans as raw material input yielding 3,681 kg of tofu. Because tofu is the sole marketable product of the system, no allocation was applied; the whey and okara generated during processing were treated as waste outputs rather than co-products, with whey and process wastewater modeled as emissions to the freshwater compartment.

The system boundary adopts a gate-to-gate approach, encompassing all unit processes from the initial washing of soybeans to final molding and packing (Figure 1). Upstream operations, including soybean farming and transportation, as well as downstream activities such as distribution, consumption, and end-of-life management, were excluded from the system boundary. This delineation is consistent with prior gate-to-gate LCA investigations of similar Indonesian tofu SMEs [10].

Figure 1. Gate-to-gate system boundary of tofu production life cycle assessment

2.2 Life Cycle Inventory Analysis

The inventory phase identified and quantified all input and output flows for each unit process relative to the functional unit [12]. Data collected include soybean inputs, water and energy consumption, fuel use, and waste outputs comprising liquid waste, solid waste, and atmospheric emissions. The inventory data are presented in Table 1. In the model, process wastewater (from washing, soaking, and molding) and the whey separated during coagulation were treated as emissions to water, carrying the organic and nutrient load (dissolved proteins, suspended solids, and nitrogen- and phosphorus-bearing compounds) released from the soybeans during processing, whereas okara and wood ash were treated as solid waste outputs.

Table 1. Life Cycle Inventory Data of Tofu Production

| Categories | Parameter | Unit | Process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Washing | Soaking | Grinding | Boiling | Filtration | Coagulation | Molding & Packing |
| Input material, Output material, Fuel, and energy | Soybeans | kg | 1400 | - | - | - | - | - | - |
|  | Water | L | 2800 | 2700 | 4200 | 8400 | - | - | 200 |
|  | Electricity | kWh | 0.60 | 0.45 | 17 | - | - | - | - |
|  | Firewood | kg | - | - | - | 800 | - | - | - |
|  | Coagulant water | L | - | - | - | - | - | 421 |  |
|  | Polyethylene plastic | g | - | - | - | - | - | - | 1656 |
| Output product, Output material | Tofu Product | kg | - | - | - | - | - | - | 3681 |
|  | Wastewater | L | 2660 | 600 | - | - | - | - | 100 |
|  | Whey | L | - | - | - | - | - | 10434 | - |
|  | Okara | kg | - | - | - | - | 1600 | - | - |
|  | Steam loss | L | - | - | - | 588 | - | - | - |
|  | Wood ashes | kg |  |  |  | 16 |  |  |  |
| Emission | CO₂ | kg | 0.51 | 0.38 | 14.45 | 1397 | - | - | - |
|  | CH₄ | kg | - | - | - | 0.0499 | - | - | - |
|  | N₂O | kg | - | - | - | 0.3744 | - | - | - |

2.3 Impact Assessment

The LCIA phase evaluates and quantifies the potential environmental impacts generated by the product system based on the compiled inventory data [5]. The CML-IA Baseline method was applied in SimaPro 10.3 across five impact categories: Global Warming Potential (GWP₁₀₀ₐ, kg CO₂ eq), Ozone Layer Depletion (ODP, kg CFC-11 eq), Freshwater Aquatic Ecotoxicity (FAET, kg 1,4-DB eq), Acidification (AP, kg SO₂ eq), and Eutrophication (EP, kg PO₄³⁻ eq). Water use and potential water scarcity were additionally assessed using the AWARE (Available WAter REmaining) method, which weights water consumption against regional water availability and therefore captures water deprivation beyond simple volumetric accounting [11]. Consistent with standard LCA practice for biomass fuels, the CO₂ released from firewood combustion was treated as biogenic and carbon-neutral; the global-warming contribution of the boiling stage therefore derives from the non-CO₂ combustion gases (CH₄ and N₂O) and from the fossil emissions embedded in the upstream firewood supply chain rather than from the combustion CO₂ itself.

2.4 Interpretation

The interpretation phase contextualized the LCIA results to identify hotspot categories and the production stages contributing the greatest burdens. Normalization was applied using the CML-IA Baseline reference values to express impact categories of different units on a common dimensionless scale, enabling their relative significance to be compared. Interpretation was confined to the gate-to-gate system boundary so that the results accurately reflect the stages exerting the greatest environmental influence [5].

3. Results and Discussion

3.1 Potential Environmental Impact of Tofu Production

The LCIA results per production stage are summarized in Table 2 and expressed per functional unit (1 kg of tofu). A notable feature is the absence of impact values for the filtration stage across all categories. This is attributable to the absence of explicit energy inputs or additional material flows for that stage: filtration operates as a passive mechanical separation step without recorded electricity consumption, so SimaPro 10.3 did not generate characterization values for it. This is consistent with the inventory in Table 1, where filtration is limited to the passage of boiled slurry and the separation of soy milk from okara residue.

Across the five impact categories, two stages emerge as recurrent hotspots: boiling, driven by firewood combustion, and coagulation, associated with the organic- and nutrient-rich whey discharged as emission to water, with grinding contributing appreciably to ecotoxicity through its electricity demand. The washing stage, by contrast, is a minor contributor (0.1–4.3%) in every CML-IA category.

Table 2. Environmental Impact of Tofu Production

| Impact Category | Unit | Total | Washing | Soaking | Grinding | Boiling | Coagulation | Molding & Packing |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Global warming | kg CO₂ eq | 1.84E-01 | 8.20E-04 | 2.60E-04 | 4.57E-03 | 1.62E-01 | 1.52E-02 | 1.60E-03 |
| Ozone layer depletion | kg CFC-11 eq | 2.53E-10 | 1.82E-12 | 5.77E-13 | 1.01E-11 | 6.87E-11 | 1.36E-10 | 3.51E-11 |
| Freshwater aquatic ecotoxicity | kg 1,4-DB eq | 1.30E-02 | 5.64E-04 | 1.79E-04 | 3.14E-03 | 4.51E-03 | 3.83E-03 | 7.46E-04 |
| Acidification | kg SO₂ eq | 1.27E-04 | 2.12E-06 | 6.72E-07 | 1.18E-05 | 4.62E-05 | 6.15E-05 | 4.78E-06 |
| Eutrophication | kg PO₄³⁻ eq | 1.75E-04 | 2.94E-06 | 9.33E-07 | 1.64E-05 | 8.42E-05 | 6.80E-05 | 2.28E-06 |

Figure 2. Environmental Impact Contributor of Tofu Production

a. Global Warming Potential (GWP)

Total GWP is 1.84 × 10⁻¹ kg CO₂ eq per kg tofu. The boiling stage is by far the largest contributor at 1.62 × 10⁻¹ kg CO₂ eq (88.0%), followed by coagulation (8.3%) and grinding (2.5%). Boiling's dominance is driven by firewood use: because the CO₂ released from firewood combustion is treated as biogenic and carbon-neutral, the stage's characterized GWP is governed by the non-CO₂ combustion gases CH₄ (0.0499 kg) and N₂O (0.3744 kg) per batch—both substantially more potent than CO₂—together with the fossil emissions embedded in the upstream firewood supply chain (procurement, processing, and transport). This finding is consistent with comparative LCA studies of wood-fired tofu production in Indonesia, where boiling was repeatedly identified as the primary emission source [6, 7]. The smaller coagulation contribution may be attributed to the upstream supply chain of the coagulant input.

b. Ozone Layer Depletion (ODP)

Total ODP is 2.53 × 10⁻¹⁰ kg CFC-11 eq, led by coagulation (53.8%) and boiling (27.2%), with molding and packing contributing 13.9%. Because none of these stages involves halogenated substances directly, the burden is allocated within the LCA framework to trace ozone-depleting and nitrogen-bearing substances embedded in the upstream supply chains of the coagulant, firewood, and packaging materials [5]. Given its negligible normalized value (Section 3.3), ODP is not an environmental priority for this system, and this pattern is reported for completeness rather than as a management target.

c. Freshwater Aquatic Ecotoxicity (FAET)

Total FAET is 1.30 × 10⁻² kg 1,4-DB eq and is the most evenly distributed category, with boiling (34.7%), coagulation (29.5%), and grinding (24.2%) together accounting for about 88% of the total. The grinding contribution is primarily associated with its electricity demand (17 kWh per batch, the largest of any stage), as freshwater ecotoxicity in the CML-IA method is strongly influenced by heavy-metal releases attributed to grid-electricity generation. The coagulation contribution may be attributed to the whey discharged as emission to water—the largest liquid output of the system (10,434 L per batch)—and to the upstream supply chain of the coagulant input, whereas the boiling contribution reflects the firewood supply chain and the aquatic deposition of combustion-derived emissions [6]. This distributed ecotoxicity profile is consistent with prior Indonesian tofu LCA studies that rank freshwater ecotoxicity among the most significant categories [9].

d. Acidification (AP)

Total AP is 1.27 × 10⁻⁴ kg SO₂ eq, led by coagulation (48.4%) and boiling (36.4%), with grinding contributing 9.3%. The boiling burden derives from SO₂ and NOₓ released during firewood combustion, the principal acidification precursors, while the coagulation contribution is primarily associated with the upstream energy and material supply chains of the coagulant input rather than with the whey itself, since acidification is governed by airborne rather than waterborne emissions [5]. Firewood-derived acidifying emissions may form acid deposition with potential consequences for soil and receiving waters near the facility.

e. Eutrophication (EP)

Total EP is 1.75 × 10⁻⁴ kg PO₄³⁻ eq, driven mainly by boiling (48.1%) and coagulation (38.9%). The coagulation contribution is primarily associated with the nutrient-rich whey discharged as emission to water, which carries nitrogen- and phosphorus-bearing compounds released during protein coagulation; if discharged untreated, such effluents can trigger algal proliferation, dissolved-oxygen depletion, and disruption of aquatic ecosystems [2]. The boiling contribution reflects the atmospheric deposition of combustion-derived NOₓ, while grinding adds a further 9.4% through its electricity demand.

3.2 Water Use Assessment

Water use was assessed with the AWARE method, which expresses the volume of water deprived from downstream users per unit consumed at a given location. The total water-scarcity impact of the production system is 569 m³ world eq (Table 3). Consistent with the GWP pattern, the boiling stage is the dominant contributor at 294 m³ world eq (51.7%). This reflects the AWARE method's sensitivity to upstream supply-chain water consumption: firewood procurement, processing, and distribution embed substantial water use that is allocated back to boiling, elevating its scarcity value beyond what direct water consumption alone would suggest. Grinding contributes the second-largest share (101 m³ world eq, 17.8%), consistent with the continuous water addition required to achieve appropriate slurry consistency. Washing (65.3 m³ world eq, 11.5%) and soaking (63 m³ world eq, 11.1%) follow, while coagulation (6.7%) and molding and packing (1.3%) are minor. The comparatively modest scarcity value of washing, despite its substantial direct water use, illustrates that water-scarcity characterization is governed not solely by withdrawn volume but by regional water availability and the embedded water footprint of upstream inputs [11].

Table 3. Water Use Assessment Results of Tofu Production

| Production Stage | Water Scarcity (m³ world eq) | Share (%) |
| --- | --- | --- |
| Boiling | 294 |  |
| Grinding | 101 |  |
| Soaking | 63 |  |
| Coagulation | 38.2 |  |
| Washing | 65.3 |  |
| Molding and Packing | 7.63 |  |
| Total | 569 | 100 |

3.3 Normalization

Normalization was performed using the CML-IA Baseline reference values to place all categories on a common dimensionless scale (Table 4). The results identify GWP as the dominant impact category (3.66 × 10⁻¹⁴), followed closely by FAET (2.50 × 10⁻¹⁴) and then EP (1.32 × 10⁻¹⁴); AP (4.51 × 10⁻¹⁵) and ODP (2.83 × 10⁻¹⁸) are comparatively minor. The normalized ranking mirrors the absolute ranking and indicates that the system's two most consequential pressures are the climate burden of firewood combustion at boiling and the freshwater-ecotoxicity burden distributed across boiling, coagulation, and grinding. This convergence is significant: it suggests that interventions at the boiling stage can simultaneously relieve the highest-ranked category (GWP) and a leading mid-ranked category (FAET), while organic liquid-waste management—particularly of the coagulation whey—addresses the remaining freshwater-related burdens (FAET and EP). AP and ODP contribute relatively minor normalized values and are therefore not the primary environmental priorities for this system.

Table 4. Normalization Results of Tofu Production

| Impact category | Normalized value |
| --- | --- |
| Global Warming Potential (GWP) | 3.66E-14 |
| Freshwater Aquatic Ecotoxicity (FAET) | 2.50E-14 |
| Eutrophication (EP) | 1.32E-14 |
| Acidification (AP) | 4.51E-15 |
| Ozone Layer Depletion (ODP) | 2.83E-18 |

3.4 Hotspot Identification and Improvement Strategies

The integrated analysis of LCIA and normalization results identifies two primary hotspots: (i) firewood combustion during boiling, which dominates GWP and water scarcity and contributes substantially to EP, AP, and FAET; and (ii) organic liquid-waste generation, principally the coagulation whey stream together with washing and soaking effluents, which drives EP and contributes substantially to FAET. The electricity demand of grinding is a secondary driver of freshwater ecotoxicity. The recommended, evidence-based improvement strategies are summarized in Table 5.

Table 5. Recommended Environmental Improvement Strategies for Tofu Production

| Improvement strategy | Description | Reference |
| --- | --- | --- |
| Anaerobic wastewater treatment / biofilter system | Reduces organic load (BOD, COD) from washing, soaking, and whey effluents; anaerobic digestion also recovers biogas as a renewable energy source, providing co-benefits for FAET and GWP. | [2] |
| Water recycling (counter-current washing) | Reuses water at the washing and soaking stages to lower clean-water consumption and minimize effluent volume and organic concentration. | [13] |
| Fuel substitution and thermal-efficiency improvement at boiling | Replaces firewood with LPG or biomethane (recovered from tofu wastewater) and adopts insulated/improved boilers and waste-heat recovery, reducing combustion emissions and the embedded water footprint. | [14] |

For the first hotspot, boiling contributes disproportionately to GWP and water scarcity owing to its reliance on firewood: combustion of 800 kg per batch releases the greenhouse gases CH₄ (0.0499 kg) and N₂O (0.3744 kg) alongside biogenic CO₂, and embeds a substantial upstream water footprint. Transitioning to cleaner energy, such as LPG or biomethane recovered from on-site biodigesters, would directly reduce these emissions while lowering the upstream water footprint of the firewood supply chain [14]. Adopting improved-combustion or thermally insulated boiling systems, together with waste-heat recovery to pre-heat process water, would further reduce specific fuel consumption.

For the second hotspot, the high-BOD/COD, nutrient-rich effluents—especially the whey generated during coagulation (10,434 L per batch), the largest liquid output of the system and the principal eutrophication driver at this stage—pose significant risks to receiving ecosystems if discharged untreated. Anaerobic treatment (e.g., biodigesters or integrated biogas wastewater units) is technically appropriate for SME-scale operations because it reduces organic loading while recovering biogas as an energy resource, simultaneously lowering FAET and EP (through effluent-load reduction) and GWP (through fuel displacement) [2]. Complementary measures include counter-current water recycling within washing and soaking to cut both water consumption and effluent generation [13], and valorization of whey as a protein-rich feed or fermentation substrate to divert high-nutrient streams away from discharge. Overall, the LCA evidence demonstrates that interventions targeting energy efficiency and liquid-waste management at the source yield substantially greater environmental benefit than conventional end-of-pipe approaches.

4. Conclusion

This study applied a gate-to-gate Life Cycle Assessment to evaluate the environmental impacts of small-scale tofu production in Semarang City using the CML-IA Baseline and AWARE methods, with a functional unit of 1 kg of tofu. Tofu production generated impacts in all assessed categories, with totals of 1.84 × 10⁻¹ kg CO₂ eq (GWP), 2.53 × 10⁻¹⁰ kg CFC-11 eq (ODP), 1.30 × 10⁻² kg 1,4-DB eq (FAET), 1.27 × 10⁻⁴ kg SO₂ eq (AP), and 1.75 × 10⁻⁴ kg PO₄³⁻ eq (EP). The boiling and coagulation stages were the principal hotspots: boiling dominated GWP (88.0%) and water scarcity (51.7%) through firewood combustion and its embedded water demand, while coagulation led ODP and AP and, together with boiling and grinding, governed freshwater ecotoxicity and eutrophication, largely through the whey discharged as emission to water. Normalization identified GWP as the most significant category (3.66 × 10⁻¹⁴), followed by FAET (2.50 × 10⁻¹⁴) and EP (1.32 × 10⁻¹⁴), and the AWARE assessment yielded a total water-scarcity impact of 569 m³ world eq, dominated by boiling.

These findings indicate that the climate burden of firewood combustion and the freshwater impacts of organic liquid waste are the most acute environmental concerns of urban tofu SMEs, and that environmental management should prioritize boiling-stage energy efficiency together with organic liquid-waste treatment and water reuse. Practical, high-leverage measures include fuel substitution and improved thermal systems, anaerobic treatment with biogas recovery, and counter-current water recycling, all of which act upstream at the source rather than end-of-pipe. By providing the first stage-resolved, AWARE-supported LCA of tofu production in Semarang City, this study contributes process-level evidence to support cleaner production in urban food SMEs. Future research should extend the boundary toward cradle-to-grave and integrate life cycle costing to confirm the economic feasibility of the proposed interventions.

References

[1] Badan Pusat Statistik 2026 Rata-Rata Konsumsi per Kapita Seminggu Beberapa Macam Bahan Makanan Penting 2007–2025 (Jakarta: Badan Pusat Statistik)

[2] Seroja R, Effendi H and Hariyadi S 2018 Tofu wastewater treatment using vetiver grass (Vetiveria zizanioides) and zeliac Appl. Water Sci. 8 2 https://doi.org/10.1007/s13201-018-0640-y

[3] Sjafruddin R, Agustang A and Pertiwi N 2022 Estimasi limbah industri tahu dan kajian penerapan sistem produksi bersih J. Ilm. Mandala Educ. 8 1229–37 https://doi.org/10.36312/jime.v8i2.2826

[4] Basuki T M, Nugroho H Y S H, Indrajaya Y, Pramono I B, Nugroho N P, Supangat A B et al. 2024 Water pollution of some major rivers in Indonesia: status, institution, regulation, and recommendation for mitigation Pol. J. Environ. Stud. 33 3515–30 https://doi.org/10.15244/pjoes/178532

[5] Chitaka T Y and Goga T 2023 The evolution of life cycle assessment in the food and beverage industry: a review Cambridge Prisms: Plast. 1 e2 https://doi.org/10.1017/plc.2023.4

[6] Rosyidah M, Masruri A and Putra R A 2020 Analysis of environmental impact with the Life Cycle Assessment (LCA) method on tofu production Int. J. Sci. Technol. Manag. 1 428–35 https://doi.org/10.46729/ijstm.v1i4.73

[7] Nugroho M E, Setyono P and Rachmawati S 2024 Analisis emisi gas rumah kaca dengan Life Cycle Assessment (LCA) dan Analytical Hierarchy Process (AHP) industri tahu J. Ilmu Lingkung. 22 1504–12 https://doi.org/10.14710/jil.22.6.1504-1512

[8] Sari I P, Kurniawan W and Sia F L 2021 Environmental impact of tofu production in West Jakarta using a life cycle assessment approach IOP Conf. Ser.: Earth Environ. Sci. 896 012050 https://doi.org/10.1088/1755-1315/896/1/012050

[9] Hartini S, Fatliana A N, Handayani N U, Wicaksono P A, Ramadan B S and Matsumoto T 2024 Life cycle assessment and life cycle cost of tofu production and its waste treatment scenario Glob. J. Environ. Sci. Manage. 10(2) 487–502 https://doi.org/10.22034/gjesm.2024.02.05

[10] Kartika Wardana S, Cucikodana Y, Almaniar S, Dwijayanti A, Maulana F and Muhlisoh N A 2024 Penilaian dampak lingkungan dengan menggunakan Life Cycle Assessment (LCA) pada industri tahu Kampung Jangkar Kulon, Cilegon Banten J. Teknol. Kim. Unimal 13 97–106 https://doi.org/10.29103/jtku.v13i2.16429

[11] Mir B A, Nurdiawati A and Al-Ghamdi S G 2025 Assessing the environmental impact of freshwater use in LCA: established practices and current methods Environ. Sci.: Water Res. Technol. 11 196–221 https://doi.org/10.1039/D4EW00641K

[12] Saavedra-Rubio K, Thonemann N, Crenna E, Lemoine B, Caliandro P and Laurent A 2022 Stepwise guidance for data collection in the life cycle inventory (LCI) phase: building technology-related LCI blocks J. Clean. Prod. 366 132903 https://doi.org/10.1016/j.jclepro.2022.132903

[13] Satar I and Permadi A 2022 Treating the tofu wastewater (TWW) using a green technology of microbial fuel cell (MFC) system Indones. J. Environ. Manag. Sustain. 6 162–7 https://doi.org/10.26554/ijems.2022.6.1.162-167

[14] Ningsih L M, Mazancová J, Hasanudin U and Roubík H 2026 Energy audits in the tofu industry: an evaluation of energy consumption towards a green and sustainable industry Environ. Dev. Sustain. 28 3737–59 https://doi.org/10.1007/s10668-024-05109-z