Life Cycle Assessment of Small-Scale Tofu Production in Semarang City: Hotspot Analysis Using CML-IA and AWARE Methods

Abdul Jabbar1, Putri Alifa Kholil1* and Desiana Fitri Awati1

1 Study Program of Environmental Science, Faculty of Mathematics and Natural Sciences, Universitas Negeri Semarang, Semarang, Indonesia

*E-mail: putrialifa@mail.unnes.ac.id

Abstract. The tofu industry is an important food-processing sector in Indonesia, but its production activities require high water and energy inputs and generate organic wastewater. This study evaluated the potential environmental impacts of small-scale tofu production in Semarang City using a gate-to-gate Life Cycle Assessment approach. The system boundary covered washing, soaking, grinding, boiling, filtration, coagulation, and molding and packaging stages. The assessment was conducted using SimaPro 10.3 with the CML-IA Baseline method, covering Global Warming Potential, Ozone Layer Depletion, Freshwater Aquatic Ecotoxicity, Acidification, and Eutrophication. Water scarcity was additionally assessed using the AWARE method. The results showed that washing and boiling were the main environmental hotspots. The total Global Warming Potential reached 5.28 × 10³ kg CO₂ eq, while Freshwater Aquatic Ecotoxicity reached 1.33 × 10³ kg 1,4-DB eq. After normalization, Freshwater Aquatic Ecotoxicity showed the highest normalized value of 2.57 × 10⁻⁹, indicating that liquid waste management is the main environmental priority. The AWARE assessment showed a total water scarcity impact of 515 m³ world eq, mainly associated with the boiling stage. Practical improvement strategies include wastewater treatment, water reuse, fuel substitution, and improved thermal efficiency. This study provides process-level evidence to support cleaner production practices in small-scale tofu industries in Semarang City.

Keywords: Life Cycle Assessment; Tofu Production; CML-IA Baseline; AWARE; Environmental Hotspot.

1. Introduction

Tofu (tahu) is a staple protein source in Indonesia, with national soybean demand for tofu and tempeh exceeding 2.66 million tonnes annually — over 93% of which is imported (Katadata, 2025). Production is dominated by small and medium-sized enterprises (SMEs) (Sari et al., 2021), yet the manufacturing process generates high-strength wastewater with COD of 8,000–11,400 mg/L and BOD of 1,000–11,880 mg/L (Nurliani et al., 2024; Hardyanti et al., 2023) — far above Indonesia's national effluent standards. Untreated discharge causes severe freshwater ecotoxicity and eutrophication, a problem documented specifically in Semarang City, where tofu SME wastewater in the Sendangguwo sub-district exceeded national BOD, COD, and TSS limits (Taufik et al., 2026), and discharge in the Pakintelan–Gunungpati area degraded surrounding groundwater quality (Sajida et al., 2025). The food processing sector — including tofu and tempeh SMEs — constitutes a significant share of Semarang's micro and small industry profile (BPS Kota Semarang, 2022), underscoring the local environmental relevance of this production system.

Although life cycle assessment (LCA) — standardized under ISO 14040 and ISO 14044 (ISO, 2006a, 2006b) — has been applied to Indonesian tofu production, key methodological gaps persist. Kurniawati et al. (2019) assessed only GWP using the IPCC method in SimaPro; Sari et al. (2021) applied CML-IA in a cradle-to-gate study but excluded freshwater ecotoxicity (FAET); Hartini et al. (2021) and Azatri et al. (2023) used CML-IA Baseline in gate-to-gate studies without reporting co-product allocation or water scarcity; and Matsumoto (2024), despite the broadest scope (cradle-to-grave, SimaPro v9.4), employed the Eco-cost 2023 endpoint method, which aggregates results into a single score and precludes midpoint hotspot identification. Critically, no prior study has applied the AWARE water scarcity method (Boulay et al., 2018) to Indonesian tofu production, nor has any LCA been conducted in Semarang City, where urban wastewater dynamics and local production practices may yield distinct environmental burden profiles from rural sites in Grobogan or Yogyakarta.

This study addresses these gaps by conducting a gate-to-gate LCA of SME-scale tofu production in Semarang City with four objectives: (1) quantify environmental impacts across five CML-IA Baseline v4.8 midpoint categories (GWP, ODP, FAET, AP, EP) using SimaPro 10.3; (2) assess water scarcity using AWARE v1.2c (Boulay et al., 2018) — the first such application to Indonesian tofu production; (3) apply mass-based co-product allocation (okara and whey) per ISO 14044; and (4) identify environmental hotspots and propose targeted improvement strategies for urban tofu SMEs.

2. Methods

This research utilized a quantitative methodology based on Life Cycle Assessment (LCA) to assess the environmental impacts of tofu production at a small and medium-sized enterprise (SME) in Semarang City. The evaluation was performed in compliance with SNI ISO 14040:2016 and SNI ISO 14044:2016, comprising four sequential phases: (1) goal and scope definition, (2) life cycle inventory analysis, (3) life cycle impact assessment, and (4) interpretation. Primary data were obtained through direct field measurements, structured interviews with the facility operator, and on-site documentation during a complete production batch. Secondary data were sourced from peer-reviewed scientific literature and the Ecoinvent v3 database within SimaPro 10.3 software [6].

2.1 Goal and scope definition

This study aims to assess the environmental impacts of tofu production at the facility level, focusing on water use, energy consumption, and waste generation at each production stage. The functional unit was established as 1 kg of tofu produced, consistent with functional units used in comparable LCA studies of tofu and soy-based food processing. All inventory data were initially collected on a per-batch basis representing one complete production day, with 1,400 kg of soybeans as raw material input yielding 3,681 kg of tofu.

The system boundary adopts a gate-to-gate approach, encompassing all unit processes from the initial washing of soybeans to final molding and packaging (Figure 1). Upstream operations, including soybean farming and transportation, as well as downstream activities such as distribution, consumption, and end-of-life management, were excluded from the system boundary. This delineation is consistent with prior gate-to-gate LCA investigations of similar Indonesian tofu SMEs [8, 9].

Figure 1. Gate-to-gate system boundary of tofu production life cycle assessment

2.2 Life Cycle Inventory Analysis

The inventory analysis phase involves the identification and collection of quantitative data on all input and output flows for each unit process within the product system, in accordance with the defined functional unit [11]. Data collected include soybean inputs, water and energy consumption, and waste outputs comprising liquid waste, solid waste, and atmospheric emissions. The inventory data are presented in Table 1.

Table 1. Life Cycle Inventory Data of Tofu Production (per batch / production day)

| Categories | Parameter | Unit | Process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Washing | Soaking | Grinding | Boiling | Filtration | Coagulation | Molding & Packing |
| Input | Soybeans | Kg | 1400 | - | - | - | - | - | - |
| material, | Water | L | 2800 | 2700 | 4200 | 8400 | - | - | 200 |
| Ouput | Electricity | kWh | 0.60 | 0.45 | 17 | - | - | - | - |
| material, | Firewood | Kg | - | - | - | 800 | - | - | - |
| Fuel, and | Coagulant water | L | - | - | - | - | - | 421 |  |
| energy | Polyethylene plastic | Kg | - | - | - | - | - | - | 1656 |
| Ouput | Tofu Product | Kg | - | - | - | - | - | - | 3681 |
| product, | Wastewater | L | 2660 | 600 | - | - | - | - | 100 |
| Ouput | Whey | L | - | - | - | - | - | 10434 | - |
| material | Okara | Kg | - | - | - | - | 1600 | - | - |
|  | Steam loss | L | - | - | - | 588 | - | - | - |
|  | Wood ashes | Kg |  |  |  | 16 |  |  |  |
| Emission |  |  | Air emissions |  |  |  |  |  |  |
|  | CO₂ | Kg | 0.51 | 0.38 | 14.45 | 1397 | - | - | - |
|  | CH₄ | Kg | - | - | - | 0.0499 | - | - | - |
|  | N₂O | Kg | - | - | - | 0.3744 | - | - | - |

2.3 Impact Assessment

The LCIA phase evaluates and quantifies the potential environmental impacts generated by the product system based on the compiled inventory data [6]. In this study, the CML-IA Baseline method was applied using SimaPro 10.3, covering five impact categories: Global Warming Potential (GWP₁₀₀ₐ, kg CO₂ eq), Ozone Layer Depletion (ODP, kg CFC-11 eq), Fresh Water Aquatic Ecotoxicity (FAET, kg 1,4-DB eq), Acidification (AP, kg SO₂ eq), and Eutrophication (EP, kg PO₄³⁻ eq). Water use and potential water scarcity were additionally assessed using the AWARE (Available WAter REmaining) method to evaluate water deprivation relative to regional water availability.

2.4 Interpretation

The interpretation phase contextualizes LCIA results to identify hotspot impact categories and the production stages contributing the greatest environmental burdens. Normalization was subsequently applied using the CML-IA Baseline reference values to compare impact categories expressed in different units on a common dimensionless scale, thereby enabling the relative significance of each impact to be determined. Interpretation was focused strictly within the gate-to-gate system boundary to ensure that results accurately reflect the stages exerting the greatest environmental influence [6].

3. Result and Discussion

3.1 Potential Environmental Impact of Tofu Production

The LCIA reveals that tofu production generates measurable environmental impacts across all five assessed categories. The quantitative results per production stage are summarized in Table 2.

Tabel 2. Enviromental Impact of Tofu Production

| Impact Category | Unit | Total | Washing | Soaking | Grinding | Boiling | Coagulation | Molding & Packing |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Global warming (GWP100a) | kg CO₂ eq | 5,28E3 | 3,56E3 | 0,945 | 35,8 | 1,63E3 | 55,8 | 5,9 |
| Ozone layer depletion (ODP) | kg CFC-11 eq | 6,11E-5 | 5,93E-5 | 2,1E-9 | 7,93E-8 | 1,07E-6 | 5,02E-7 | 1,29E-7 |
| Fresh water aquatic ecotoxicity | kg 1,4-DB eq | 1,33E3 | 1,22E3 | 0,651 | 24,6 | 70,6 | 14,1 | 2,75 |
| Acidification | kg SO₂ eq | 4,59 | 3,53 | 0,00245 | 0,0925 | 0,723 | 0,226 | 0,0176 |
| Eutrophication | kg PO₄³⁻ eq | 9,1 | 8,3 | 0,0034 | 0,128 | 0,409 | 0,25 | 0,00838 |

Based on the results presented in Table 2, a notable feature of the results is the absence of impact values for the filtration stage across all categories. This is attributable to the absence of explicit energy inputs or additional material flows entered the inventory model for that stage — the filtration process in this facility operates as a passive mechanical separation step without recorded electricity consumption, and consequently, SimaPro 10.3 did not generate characterization values for it. This finding is consistent with the inventory structure presented in Table 2, where filtration inputs are limited to the passage of boiled slurry and the separation of soy milk from okara residue without additional resource inputs.

a. Global Warming Potential (GWP)

Total GWP amounts to 5.28 × 10³ kg CO₂ eq. The washing stage is the largest contributor at 3.56 × 10³ kg CO₂ eq (approximately 67.4%), followed by the boiling stage at 1.63 × 10³ kg CO₂ eq (approximately 30.9%). The dominance of the washing stage is attributed to the large volumes of water consumed during soybean cleaning, which indirectly require energy for water supply and distribution, thereby generating greenhouse gas emissions. In addition, organic-laden washing wastewater has the potential to generate methane (CH₄) if discharged without treatment. The boiling stage contributes significantly to GWP owing to the combustion of firewood, which releases CO₂, CH₄, and N₂O, all of which are potent greenhouse gases with long-term atmospheric warming effects.

b. Ozone Layer Depletion (ODP)

Total ODP registers 6.11 × 10⁻⁵ kg CFC-11 eq, with the washing stage dominating at 5.93 × 10⁻⁵ kg CFC-11 eq (approximately 97.1%). This impact is not attributed to water consumption per se, but rather to the energy-intensive infrastructure required for water supply and distribution within the production chain, which generates indirect emissions linked to ozone-depleting substances. Although this category contributes the least in normalized terms, its disproportionate concentration in the washing stage highlights the indirect environmental burden of high water use.

c. Fresh Water Aquatic Ecotoxicity

Total FAET reaches 1.33 × 10³ kg 1,4-DB eq, with the washing stage as the primary contributor at 1.22 × 10³ kg 1,4-DB eq (approximately 91.7%). The large volumes of organic-laden wastewater generated during soybean washing contain residual organic matter, dissolved solids, and impurities that constitute a significant toxicity risk to freshwater organisms if discharged without adequate treatment. The boiling stage contributes 70.6 kg 1,4-DB eq (approximately 5.3%) through the atmospheric deposition of combustion byproducts, including nitrogen oxides (NOₓ) and sulfur oxides (SOₓ), into aquatic environments.

d. Acidification

Total Acidification impact is 4.59 kg SO₂ eq. The washing stage accounts for 3.53 kg SO₂ eq (approximately 76.9%), associated with SO₂ and NOₓ emissions from energy generation required for water provision. The boiling stage contributes 0.723 kg SO₂ eq (approximately 15.8%) through firewood combustion, which releases acidifying gaseous precursors that may react with atmospheric moisture to form acid deposition, potentially degrading both soil and water quality in the surrounding environment.

e. Eutrophication

Total Eutrophication impact is 9.10 kg PO₄³⁻ eq, with the washing stage dominant at 8.30 kg PO₄³⁻ eq (approximately 91.2%). This impact is driven primarily by nutrient-rich washing wastewater that contains nitrogen and phosphorus compounds derived from soybean residues. When discharged into receiving water bodies without treatment, these compounds can trigger excessive proliferation of algae and aquatic microorganisms, leading to depletion of dissolved oxygen and disruption of aquatic ecosystem function. The boiling stage contributes 0.409 kg PO₄³⁻ eq (approximately 4.5%) through atmospheric deposition of NOₓ emissions generated during firewood combustion.

3.2 Water Use Assessment

Water use was assessed using the AWARE method to quantify potential water scarcity, expressed as the volume of water deprived from downstream users per unit of water consumed at a given location. The total water scarcity impact for the entire production system amounts to 515 m³ world eq.

Table 3. Water Use Assessment Results of Tofu Production

| Production Stage | Water Scarcity (m³ world eq) | Share (%) |
| --- | --- | --- |
| Boiling | 294 | 57.09 |
| Grinding | 101 | 19.61 |
| Soaking | 39.3 | 7.63 |
| Coagulation | 38.2 | 7.42 |
| Washing | 35.8 | 6.95 |
| Molding and Packaging | 7.63 | 1.48 |
| Total | 515 | 100 |

The boiling stage is the dominant contributor (approximately 57.1%), reflecting both direct water consumption during soybean slurry cooking and the substantial upstream water demand embedded in the firewood supply chain. Within the AWARE framework, the environmental burden is determined not solely by the volume of water consumed, but also by the water intensity of the upstream energy system and associated supply chains. The grinding stage contributes the second largest share (approximately 19.6%), as continuous water addition is required throughout this process to achieve the appropriate slurry consistency and extraction efficiency.

3.3 Normalization

Normalization was performed using the CML-IA Baseline reference values to place all impact categories on a common dimensionless scale, enabling direct and meaningful comparison of relative environmental significance across categories with different units.

Table 4. Normalization Results of Tofu Production

| Impact Category | Normalized Value |
| --- | --- |
| Global warning potential (GWP100a) | 1,05E-9 |
| Ozon layer depletion (ODP) | 6,84E-13 |
| Fresh water aquatic ecotoxity | 2,57E-9 |
| Acidification | 1,63E-10 |
| Eutrophication | 6,89E-10 |

Normalization confirms that Fresh Water Aquatic Ecotoxicity (FAET) is the dominant environmental impact category in this production system, indicating that tofu production exerts the greatest relative pressure on freshwater ecosystems. This finding underscores that organic liquid waste management, particularly the treatment of effluents from the washing, soaking, and coagulation stages, constitutes the most critical environmental challenge in the analyzed production system.

Although Global Warming Potential frequently receives primary attention in environmental sustainability discourse, its normalized contribution ranks second in this study, indicating that freshwater pollution represents a more acute environmental concern at the facility scale assessed. Eutrophication, which ranks third in normalized impact, further reinforces the urgency of addressing nutrient-laden wastewater streams as a priority intervention. Acidification and Ozone Layer Depletion, while present, contribute relatively minor normalized values, suggesting that their management, though important, does not constitute the primary environmental priority for this production system.

3.4 Hotspot Identification and Improvement Strategies

The integrated analysis of LCIA results and normalization outcomes identifies two primary environmental hotspots in the tofu production system, namely the generation of organic liquid waste across the washing, soaking, and coagulation stages, and the combustion of firewood during the boiling stage. These hotspots correspond, respectively, to the dominant drivers of Fresh Water Aquatic Ecotoxicity and Eutrophication (ranked first and third in normalized impact), and of Global Warming Potential and water scarcity.

With respect to the first hotspot, the organic-laden wastewater streams generated during washing, soaking, and coagulation are characterized by high BOD, COD, and nutrient loads [13, 14], and their uncontrolled discharge poses significant risks to receiving aquatic ecosystems. The implementation of anaerobic treatment systems, such as biodigesters or integrated biogas wastewater treatment units (IPAL biogas), represents an effective and technically appropriate intervention for SME-scale operations, as such systems simultaneously reduce organic loading while generating biogas as a recoverable energy resource. Complementary measures include the application of constructed wetland or biofilter systems for tertiary nutrient removal prior to effluent discharge, as well as counter-current water recycling and recirculation within the washing and soaking stages to reduce both volumetric water consumption and wastewater generation. Furthermore, the valorization of whey, for instance as a protein-rich substrate for animal feed or as a feedstock for fermentation processes, offers a viable pathway for diverting high-nutrient liquid streams away from the wastewater treatment system, thereby reducing the overall nutrient load requiring treatment [10].

With respect to the second hotspot, the boiling stage contributes disproportionately to GWP and water scarcity owing to its reliance on firewood as the primary thermal energy source. The combustion of 800 kg of firewood per production batch releases 1,397 kg CO₂, 0.0499 kg CH₄, and 0.3744 kg N₂O, collectively representing a substantial greenhouse gas burden. Transitioning to cleaner energy sources, such as liquefied petroleum gas (LPG) or biomethane derived from on-site biodigesters, would directly reduce these emissions while also decreasing the upstream water footprint associated with firewood supply chains. In parallel, the adoption of improved combustion systems or thermally insulated boiling tanks would reduce specific fuel consumption per unit of output through minimization of heat loss, while waste heat recovery from boiler exhaust could be directed toward pre-heating process water at upstream stages, thereby improving overall energy efficiency. The LCA-based analysis demonstrates that improvement interventions targeting liquid waste management and energy use efficiency at the source generate substantially greater environmental benefits than conventional end-of-pipe approaches [10, 15]..

4. Conclusion

This study applied a gate-to-gate Life Cycle Assessment to evaluate the environmental impacts of small-scale tofu production in Semarang City using the CML-IA Baseline and AWARE methods. The results showed that tofu production generated impacts in all assessed categories, with total values of 5.28 × 10³ kg CO₂ eq for GWP, 6.11 × 10⁻⁵ kg CFC-11 eq for ODP, 1.33 × 10³ kg 1,4-DB eq for FAET, 4.59 kg SO₂ eq for acidification, and 9.10 kg PO₄³⁻ eq for eutrophication.

The washing and boiling stages were identified as the main environmental hotspots. Washing contributed dominantly to freshwater aquatic ecotoxicity and eutrophication due to organic wastewater generation, while boiling contributed substantially to GWP and water scarcity because of firewood combustion and high water demand. Normalization results indicated that freshwater aquatic ecotoxicity was the most significant impact category, with a normalized value of 2.57 × 10⁻⁹. The AWARE assessment showed a total water scarcity impact of 515 m³ world eq, mainly contributed by the boiling stage.

Environmental improvement should therefore prioritize wastewater treatment, water reuse, and energy efficiency in the boiling process. Practical measures such as anaerobic treatment, water recycling, fuel substitution, and improved thermal systems can reduce the environmental burden of tofu production. Overall, this study provides useful evidence to support cleaner production practices and environmental management for small-scale tofu industries in Semarang City..

References

[1]Ghisellini, P., Cialani, C. and Ulgiati, S. (2016) ‘A review on circular economy: The expected transition to a balanced interplay of environmental and economic systems’, Journal of Cleaner Production, 114, pp. 11–32. Available at: https://doi.org/10.1016/j.jclepro.2015.09.007.

[2] Dewi, A. N., & Setiawan, D. (2024). Bisnis Kuliner ( Studi Kasus CV . Gehu Extra Pedas Chili Hot ). Jurnal Manajemen Dan Bisnis Islam, 1(1), 5–23.

[3] Rosyidah, M., Masruri, A., & Putra, R. A. (2020). Assessment ( LCA ) Method On Tofu Production. International Journal of Science, Technology & Management, 1(4), 428–435. https://ijstm.inarah.co.id/index.php/ijstm/article/view/73/62

[4] Sjafruddin, R., Agustang, A., & Pertiwi, N. (2022). Estimasi Limbah Industri Tahu Dan Kajian Penerapan Sistem Produksi Bersih. Jurnal Ilmiah Mandala Education, 8(2), 1229–1237. https://doi.org/10.36312/jime.v8i2.2826

[5] Islami, M. C. P. A., & Harnaningrum, R. N. (2025). Integration of Waste Management and Environmental Impact Assessment for Sustainable Manufacturing in Sukolego Tofu Production. 06(2), 71–77.

[6] Hauschild, M. Z., Rosenbaum, R. K., & Olsen, S. I. (2017). Life Cycle Assessment: Theory and Practice. In Life Cycle Assessment: Theory and Practice. https://doi.org/10.1007/978-3-319-56475-3

[7] Chitaka, T. Y., & Goga, T. (2023). The evolution of life cycle assessment in the food and beverage industry: A review. Cambridge Prisms: Plastics, 1, 1–6. https://doi.org/10.1017/plc.2023.4

[8] Nugroho, M. E., Setyono, P., & Rachmawati, S. (2024). Analisis Emisi Gas Rumah Kaca dengan Life Cycle Assessment (LCA) dan Analytical Hierarchy Process (AHP) Industri Tahu. Jurnal Ilmu Lingkungan, 22(6), 1504–1512. https://doi.org/10.14710/jil.22.6.1504-1512

[9] Kartika Wardana, S., Cucikodana, Y., Almaniar, S., Dwijayanti, A., Maulana, F., & Muhlisoh, N. A. (2024). Penilaian Dampak Lingkungan Dengan Menggunakan Life Cycle Assesment (Lca) Pada Industri Tahu Kampung Jangkar Kulon, Cilegon Banten. Jurnal Teknologi Kimia Unimal, 13(2), 97–106. https://doi.org/10.29103/jtku.v13i2.16429

[10]Bjørnbet, M. M., & Vildåsen, S. S. (2021). Life cycle assessment to ensure sustainability of circular business models in manufacturing. Sustainability (Switzerland), 13(19). https://doi.org/10.3390/su131911014

[11]Saavedra-Rubio, K., Thonemann, N., Crenna, E., Lemoine, B., Caliandro, P., & Laurent, A. (2022). Stepwise guidance for data collection in the life cycle inventory (LCI) phase: Building technology-related LCI blocks. Journal of Cleaner Production, 366(June), 132903. https://doi.org/10.1016/j.jclepro.2022.132903

[12] Rasyid, M., & Anggriani, R. (2024). Penerapan Life Cycle Assessment (LCA) Pada Proses Produksi Minyak Kayu Putih Di Desa Sawa-Namlea. Journal Of Social Science Research, 4(3), 18970–18984.

[13]Seroja, R., Effendi, H. and Hariyadi, S. (2018) ‘Tofu wastewater treatment using vetiver grass (Vetiveria zizanioides) and zeliac’, Applied Water Science, 8(1), pp. 1–6. Available at: https://doi.org/10.1007/s13201-018-0640-y.

[14]Satar, I. and Permadi, A. (2022) ‘Indonesian Journal of Environmental Management and Sustainability Treating The Tofu Wastewater (TWW) using a Green Technology of Microbial Fuel Cell (MFC) System’, Indonesia Journal of Environmental Management and Sustainability, 6(1), pp. 162–167.

[15]Apriyanti, D., Pratikno, F.A. and Hertadi, C.D.P. (2025) ‘Penilaian Dampak Lingkungan Menggunakan Life Cycle Assessment (Lca) Pada Proses Produksi Tempe Di Siks’, Jurnal Mitra Teknik Industri, 4(1), pp. 19–27. Available at: https://doi.org/10.24912/jmti.v4i1.34600.